## ROCKAWAY BOROUGH SCHOOL DISTRICT

103 EAST MAIN STREET                                                                                    MR. MICHAEL KLEIN

ROCKAWAY, NEW JERSEY 07866                                         SUPERVISOR OF BUILDINGS AND GROUNDS

TEL:  973-625-8601

FAX: 973-625-7355

LEAD-BASED PAINT MANAGEMENT AND RESPONSE PLAN FOR UNDER 6 SQ. FT.

When there is a paint chip or damage to an area that may contain or is suspected to contain a lead based paint product the following protocol will be used for any damage that is less than six square feet.

1.  The immediate area will be isolated from foot traffic and the building principal, maintenance/custodial personnel, and the Supervisor of Buildings and Grounds shall be notified of the situation.

2.  Custodial or Maintenance personnel shall bring a H.E.P.A. vacuum to the location and if the particles/dust/paint chips are small enough for the vacuum they can be cleaned up in such manner.

For larger pieces the area or debris shall be wet down with either a soap and water solution or an appropriate cleaning agent such as a neutral floor cleaner.

3.  The larger pieces will be swept only when saturated with a cleaning solution and put into a minimum 6 mil plastic garbage back and the garbage back either double knotted or “goosenecked” at the opening.  The sealed back can be discarded in the dumpster as ordinary trash at this point.

4.  The floor or any horizontal surface within 5 feet of the damaged area shall be moped/cleaned with a neutral cleaning solution and immediately after shall be re-moped or cleaned with just plain water.

5.  The damaged area can now be repaired using regular latex paint and standard repair practices can be applied.

6.  When scraping walls or painted areas that may have lead-based paint care shall be taken to keep the work as minimal as possible and use lead-based paint approved methods of removal.

                - 12T341